What is a Network?

A network is a system of computer devices connected together so that equipment and information can be shared. Networks allow computers to communicate with each other and exchange data efficiently.

The complete architecture of a network includes several key components:

- Hardware components (computers, routers, switches)

- Software (operating systems, applications)

- Connectivity (cables, wireless signals)

- Network technologies and communication protocols

- Transmission media (how data travels)

- Network infrastructure (the physical setup)

- Network management and security policy

Types of Networks by Size

Networks are classified by their geographical coverage:

PAN (Personal Area Network)

A network for short distances, typically connecting personal devices near one person.

- Wired: USB connections

- Wireless: Bluetooth, NFC

LAN (Local Area Network)

A network for medium-range distances, usually covering a building or a home.

- Wired: Ethernet, metallic cables

- Wireless: Wi-Fi (radio waves), visible light communications

MAN (Metropolitan Area Network)

A network for city-wide coverage, connecting Wi-Fi hotspots across a metropolitan area.

- Wired: Fibre-optic cables, Metro Ethernet

- Wireless: Wi-Fi, fixed WiMAX (microwave transmission)

WAN (Wide Area Network)

A network for long distances, such as the Internet.

- Wired: Fibre-optic cables, N-Gigabit Ethernet

- Wireless: Mobile WiMAX, cellular networks (3G, 4G LTE, 5G)

GAN (Global Area Network)

A network used for earth-satellite communication.

- Wired: Not applicable

- Wireless: Navigation systems (GPS, Galileo)

Network Topologies

Topology refers to the physical layout or arrangement of a network. There are three principal topology types:

Star Topology

Each node is directly connected to a central connection point (switch or router). All computers and network devices are connected by separate LAN cables to this central point. There can also be an extended star topology with multiple central points.

Bus Topology

All computers are connected to a main cable or bus in a linear arrangement.

Ring Topology

All computers are connected in a closed loop or circle, where data travels in one direction around the ring.

Communication Models

There are two widely used models of communication in networks:

Client-Server Model

This model requires a separate server – a central computer which stores and distributes information to other computers (clients). Clients are connected to the server, and the remote server provides services to clients.

How it works:

- The client sends a request to the server

- The server processes the request and sends back a response

Peer-to-Peer Model

In this model, each node can act as both a client and a server, so it can request services and also provide services to other nodes. There is no central server – all computers have equal status.

Advantage: Flexible and decentralized
Disadvantage: There might be security issues in this model since there's no central control

Network Protocols

A network protocol is a set of rules or standards of communication that different devices and systems use to communicate with each other. Computers need to use the same protocol or standard of communication to understand each other.

The Internet uses network protocols based on the TCP/IP four-layer protocol suite.

Network Models: OSI vs TCP/IP

To simplify and better understand network communication, two important theoretical frameworks were developed:

The OSI Model (Open System Interconnection)

The OSI Reference model is a theoretical framework using seven abstract layers – each layer has different purposes and handles different parts of the communication process. This model helped develop standards and contributed to interoperability between different systems.

The 7 Layers of OSI:

| Layer | Name | Function |
| --- | --- | --- |
| 7 | Application Layer | The top layer providing interaction between applications and users |
| 6 | Presentation Layer | Formats data, translates languages |
| 5 | Session Layer | Performs and controls connections, manages dialogs between computers (called sessions) |
| 4 | Transport Layer | Provides transfer of data divided into segments and flow control |
| 3 | Network Layer | Performs routing to select the best route through the network |
| 2 | Data-Link Layer | Controls transmission of data |
| 1 | Physical Layer | Transmits data bits (e.g., through Ethernet cables) |

The TCP/IP Model

The TCP/IP model is divided into four layers and is the practical model used by the Internet. Computer networks use packet switching for data transmission.

The 4 Layers of TCP/IP:

| Layer | Name | Function | Example Protocols |
| --- | --- | --- | --- |
| 4 | Application Layer | The top-most layer related to layers 7-5 of the OSI model. Provides communication functions. | Email: SMTP, POP3, IMAP Web: HTTP, HTTPS, DNS File sharing: FTP, SMB, NFS |
| 3 | Transport Layer | Provides message segmentation and transportation. | TCP, UDP |
| 2 | Internet Layer | Enables logical addressing to deliver data packets, traffic routing and traffic control. | IPv4, IPv6, ICMP, ICMPv6 |
| 1 | Network Access Layer (also called Network Interface Layer) | Combines the physical and data link layer of the OSI model. | Ethernet, ARP, Wi-Fi |

Comparing OSI and TCP/IP Models

Similarities:

- Both models describe how networks communicate

- Both use layers to organize network functions

- Both help understand the communication process between computers

Key Differences:

| Feature | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Number of layers | 7 layers | 4 layers |
| Type | Theoretical framework | Practical model (actually used) |
| Development | Designed as a reference standard | Developed from real Internet protocols |
| Top layers | Application, Presentation, and Session are separate | All combined into one Application Layer |
| Bottom layers | Physical and Data-Link are separate | Combined into Network Access Layer |
| Usage | Teaching and understanding concepts | Running the actual Internet |

How the layers relate:

- TCP/IP Layer 4 (Application) = OSI Layers 7, 6, 5 (Application, Presentation, Session)

- TCP/IP Layer 3 (Transport) = OSI Layer 4 (Transport)

- TCP/IP Layer 2 (Internet) = OSI Layer 3 (Network)

- TCP/IP Layer 1 (Network Access) = OSI Layers 2, 1 (Data-Link, Physical)

Key Vocabulary

- Protocol – a set of rules for communication

- Packet – a unit of data transmitted over a network

- Router – a device that directs data packets between networks

- Switch – a device that connects devices within a single network

- Node – any device connected to a network

- Topology – the physical or logical arrangement of network devices

- Segmentation – dividing data into smaller pieces for transmission

- Routing – selecting the best path for data to travel through a network

- Interoperability – the ability of different systems to work together