Transfer media used in data networking

There are 3 basic types of transfer media used in data networking:

- metallic cables

- optical cables

- wireless media

Metallic cables

Data transmission is carried out by electrical signals which are transmitted through a copper

conductor. Metallic cables are for example Unshielded Twisted Pair (UTP) cable, phone line or

coaxial cable. UTP cable is the most frequently used transfer medium in Local Area Networks (LAN).

The advantages of metallic cables are high transfer rate, cheap components for installation

(connectors, tools...) and the price for the transfer medium itself.

The disadvantage of unshielded metallic cables (e.g. UTP cables) is that there can be some failures

because of electromagnetic interference which can be caused by power circuits, other collateral data

cables and electrical machines. Another disadvantage can be the limited range of communication

through a metallic cable (in case of a UTP cable it is 100 m). Finally, a disadvantage can also be the

need to install the cables which is time consuming and financially demanding with larger nets.

Optical cables

Data transfer is carried out by means of light signals which are transferred through glass or plastic

fibres. The source of optical signal can be a laser diode or LED (light-emitting diode).

An optical fibre consists of:

- the core – made of glass or plastic, serves as the carrier of a light signal

- the cladding – made of glass or plastic, prevents the light ray from escaping from the core

Optical fibres are classified (according to the way the light signal is transferred through the core) as

two basic types:

- single-mode optical fibres

- multi-mode optical fibres

Single-mode fibres

Cable core and cladding are both made of glass. Only one ray is transferred through the optical fibre

(core) and it is transferred through its centre, as you can see in the picture below. The source of

radiation is a laser diode and we use the wavelength of 1310 nm and 1550 nm. The diameter of the

core is 8–10m and the diameter of the coat is 125m. Single-mode fibres are used for long-distance

transfers – as backbone connections.

Multi-mode optical fibre

The core and the cladding are made of glass or plastic. Several light rays are transferred through the

optical fibre (its core) and enter the core under various angles, as you can see in the picture below.

The source of radiation is a light-emitting diode and there is used a wavelength of 850nm. The

diameter of the core is bigger than in a single-mode optical fibre, and that is more than 50m

(typically 62.5 m). The cladding’s diameter is 125m. Multi-mode optical fibres have bigger losses,

so they are used for shorter distance transfers – connections between buildings or distributions

inside buildings (max 2 km).

Compared to metallic cables, optical fibres are immune to electromagnetic interference and have a

smaller attenuation (i.e. transmission loss), so they enable transmitting the signal on longer

distances with no need to regenerate the signal – by order of tens of kilometres.

However, optical cables are more expensive than metallic cables for the same distance, although

they provide higher transmission rates. We also need larger range of knowledge and more expensive

appliances for pigtailing and connecting the cables and we must be more careful while doing this.

Wireless media

Wireless technologies transmit data by means of radio or microwave frequencies. The transmission

proceeds between an access point and a wireless network card. The access point concentrates

signals from users and connects wireless users. At the same time it is connected to a classical LAN

through a UTP cable. Wireless network card allows wireless communication of a host with the access

point.

The advantage of wireless transmission is that there are not any cables or data lines needed for the

transmission. The transmission proceeds in an open environment which reduces costs and shortens

time for building the network and also enables mobility of the hosts.

On the other hand, there are some disadvantages. Wireless transmission is prone to interference. Its

source can be wireless phones, fluorescent lamps, microwave oven or other wireless networks – see

the picture below. Another disadvantage is easy access of an unauthorised person to the data

transmission – that is why it is necessary to secure the network against unauthorised entering of

devices or users (see the picture below). Lastly, barriers on the way reduce the range and quality of

the signal.

Vocabulary

access point přístupový bod

angles úhly

attenuation útlum

backbone connections páteřní spoje

cladding plášť

coat ochranný plášť

coaxial koaxiál(ní)

collateral paralelní, souběžný

copper conductor měděný vodič

core jádro

diameter průměr

fibre (fiber [AmE]) vlákno

light-emitting vydávající světlo

multi-mode mnohobodová

pigtailing napojování, konektorování

ray paprsek

single-mode jednovidová

transfer media přenosová média

transfer rate přenosová rychlost

Unshielded Twisted Pair nestíněná kroucená dvoulinka

wavelength vlnová délka

wireless network card bezdrátová síťová karta

Practice

1) Match each term with its description.

1. Metallic cables

2. Single-mode optical fibres

3. Multi-mode optical fibres

4. Access point

5. Wireless network card

A) network interface controller which connects to a radio-based computer network

rather than a wire-based network

B) enable data transmission carried out by electrical signals which are transmitted

through a copper conductor

C) is a device that allows wireless devices to connect to a wired network using Wi-Fi, or

related standards

D) Optical cables used for data transfer. Cable core and cladding of the fibre are both

made of glass. Only one ray is transferred through the optical fibre (core) and it is

transferred through its centre.

E) Optical cables used for data transfer. The core and the cladding are made of glass or

plastic. Several light rays are transferred through the optical fibre (core) and enter

the core under various angles.

2) Name 3 basic types of transfer media.

3) What kind of fibre does each picture show?

4) What are advantages and disadvantages of wireless transmission